Quick look pumping sheet

1 ¾” handline:

Fog tip @ 125gpm                 15/16” SB tip @185gpm

150’-                    120psi (400sqft)                          120psi (500sqft)

200’-                    140psi (400sqft)                          140psi (500sqft)

2” Handline:

Fog tip @ 210gpm                 1 1/8” SB tip @265gpm

200’-                    160psi (600sqft)                          160psi (800sqft)

Euless load (1 line) 200gpm w/fog, 266gpm w/smooth

(105psi+5psi per 100’ of 3”)

100’        150’         200’         250’         300’         350’          400’        450’         500’

110psi   112.5psi  115psi   117.5psi   120psi    122.5psi    125psi    127.5psi   130psi

Euless load (2 lines) 400gpm w/fog, 532gpm w/smooth

(105psi+20psi per 100’ of 3”+10psi)

100’       150’        200’      250’      300’       350’     400’     450’      500’

135psi    145psi    155psi  165psi  175psi  185psi  195psi 205psi  215psi

Blitzfire- 500gpm

(115psi+25psi per 100’ of 3”)

100’        150’         200’         250’           300’       350’         400’         450’          500’

140psi  152.5psi   165psi    177.5psi   190psi    202.5psi    215psi    227.5psi  240psi

Deckgun (tank water) @400gpm:

Fog tip: 85psi           1 ¼” SB tip: 125psi

Deckgun (water supply)

Fog tip@500gpm:       1 ¼”SB tip@400gpm:      2”tip@700gpm & 1050gpm:                                                80psi                                              125psi                                            80psi           150psi

Highrise load:

2.5” Handline 1 3/16” SB tip

40psi NP @ 265gpm                         50psi NP @ 296gpm

200’-                68psi                                                      85psi

Handline smooth tip pressure:      Masterstream smooth tip pressure:

50psi                                                                      80psi

General known facts:

Needed Fire flow formula (gpm): (Length x Width) ÷ 3

Red Lines/Forestry hose:                                   Sprinklers and Standpipes:

200psi at all nozzle settings                                      150psi maintained

Elevation loss:                                                       Max psi on a hoseline:

(1/2 of elevation) or                                                   270psi max on < 3”

number of floors -1 X 5                                              180psi max on LDH

Apliance loss:                                                     Smooth bore GPM formula

+10 psi if flow is > 350gpm                                       29.7 x D² x √NP

Supply pumping to Apparatus:

Total GPMs of attack engine + FL of 5” (.08Q²) + 20psi for residual

Foam eductuor:

1 ¾” nozzle at 95gpm, pump 200psi plus FL of hose.